[image: image1.png]5vDC






  [image: image3.jpg]Hub far Versatile Seience &

Technologies






https://www.youtube.com/watch?v=QSOMQpUkCcM&t=127s
Vehicle Speed Control and Accident-Avoidance System
The project aims in designing an intelligent system which helps in avoiding accidents by alerting and controlling the speed if vehicle detect the obstacle.

In day-to-day life road accidents goes on increasing due to varies reasons. Some road accidents may cause due to chasing between two vehicles. We avoid these kinds of accidents by providing some precautionary methods using ultrasonic sensors.  Ultrasonic sensors generate high frequency sound waves and evaluate the echo which is received back by the sensor. These sensors calculate the time intervals between sending the signal and receiving the echo to determine the distance to an object.
In this project we are using two ultrasonic sensors which we mount the front and back side of the vehicle. When the vehicle detects the obstacle through sensors, the distance of the obstacle will be display on LCD display. If the distance of the obstacle is <30 cm, the vehicle will stop automatically and activate the buzzer for obstacle alerts. Relay is use to cut off the power supply of the motor. DC motors along with l293d motor diver is uses to moves the vehicle.
The main controlling device of the system is a Microcontroller. Ultrasonic sensor, LCD, buzzer and DC motor are interfaced to Microcontroller. The ultrasonic sensor continuously feds the distance input to Microcontroller. The microcontroller gives an alarm if distance is less than predefined. The speed is automatically controlled in such situations. The distance can be continuously monitored on LCD display. To perform the intelligent task, Microcontroller is programmed using embedded ‘C’ language.                                                            

Features:
1. Automatic braking system.

2. Object identification through ultrasonic sensor

3. Continuous monitoring of distance on LCD

4. Buzzer based alarming system.
The project makes us learn about:
1. Ultrasonic sensor characteristics.

2. DC motor working principle and need for a motor driver.

3. Embedded C programming.

4. PCB designing.

The Major Building blocks of this project are: 

1. Regulated Power Supply.

2. PIC Microcontroller.

3. Ultrasonic sensors.

4. DC motor with driver.

5. LCD with driver.

6. Relay.

7. LED Indicators.

8. Buzzer with driver.

9. Crystal Oscillator.

Software’s used:

1. PIC-C compiler for Embedded C programming.

2. PIC kit 2 programmer for dumping code into Micro controller.

3.  Express SCH for Circuit design.

Regulated Power Supply:
[image: image4.jpg]



 BLOCK DIAGRAM:

[image: image2.jpg]Vehicle speed control and accident avoidance system

Regulated power supply

Two
Ultrasonic [
Sensors

LED indicators




www.hvstechnologies.in                                                                            Ph: +91 9603140482

Ameerpet:      #A-7, 2nd floor, Eureka court, Above KS bakers, Ameerpet, Hyderabad – 73.


[image: image3.jpg]