[image: image1.png]5vDC




  [image: image3.jpg]Hub far Versatile Seience &

Technologies






Coine Based Mobile Charging Using Arduino

The growth of mobile phone market is phenomenal in recent years and the need for charging the mobile battery is required anytime and anywhere. In many developing countries the grid power is not available for few hours to several hours on daily basis especially in semi urban and rural areas where the mobile phones are the essential communication device. A coin based universal mobile battery charger is designed and developed in this project. This device is like a vending machine for mobile battery charging and the user has to plug the phone into the adapter and insert a coin for charging at a constant current for a definite duration. 

The mobile battery charger starts charging a mobile connected to it when a coin is inserted at the coin insertion slot at the input stage. A sensor attached to the coin insertion slot accepts the coin into the battery charging unit and start charging the mobile battery for a specific period controlled by the software of the microcontroller. LCD is used to display the charging time of the mobile and the duration of the charge of the mobile phone. 

In this project we are using GD007 coin sensor to insert the coin. When coin is inserted in the coin sensor, the coin sensor gives input to the microcontroller. Then microcontroller drives the Relay and the mobile phone is charged at particular duration of time. After completion of the time, microcontroller will turn OF the relay automatically so, mobile won’t get charge. If the coin is not placed in the box, the mobile will not charge.
The objectives of the project include: 

1. Sensing of the coin through gd007 coin sensor.

2. Visible alerts using LCD.

3. This system is very useful for mobile battery charging.
The major building blocks of this project are:

1. Regulated Power Supply.

2. Arduino UNO Microcontroller.
3. Coin sensing module
4. LCD display with driver.

5. Relay.

6. Three pin sockets.

Software’s used:

1. Embedded C programming.

2. Arduino IDE programmer for dumping code into Micro controller.

3. Express SCH for Circuit design.

Regulated power supply:

[image: image4.jpg]



Block Diagram:

[image: image2.jpg]BLOCK DIAGRAM

Regulated power supply

._‘

- Arduino
UNO
Micro

controller

Stepdown
Transformer

GD007

coin sensing —)
module

deriver




www.hvstechnologies.in                                                                            Ph: +91 9603140482

Ameerpet:      #A-7, 2nd floor, Eureka court, Above KS bakers, Ameerpet, Hyderabad – 73.


[image: image3.jpg]