[image: image1.jpg]BLOCK DIAGRAM

Step down transformer

DC
tor

Switch

Relay

Charging
circuit  [—3)] Relay |

Voltage &
current
sensor

ADC

Converter

PIC Micro
controller

Two
buttons

[ LM35 Temperature sensor

LCD display
with driver

Cooling fan
with driver

Reset
Button

Crystal
oscillator

LED indicator

{T



  [image: image2.jpg]Hub far Versatile Seience &

Technologies






EV BMS with Charger Monitoring and Fire Protection
https://www.youtube.com/watch?v=ILYYLbveuG8
                   The use of green energy is becoming increasingly more important in today’s world. Therefore, electric vehicles are currently the best choice for the environment in terms of public and personal transportation. Because of its high energy and current density, lithium-ion batteries are widely used in electric vehicles. Unfortunately, lithium-ion batteries can be dangerous if they are not operated within their Safety Operation Area (SOA). Therefore, a battery management system (BMS) must be used in every lithium-ion battery, especially for those used in electric vehicles.

A battery management system (BMS) is proposed which is used for electronic vehicle that manages a rechargeable battery (cell or battery pack), such as by protecting the battery from operating outside its safe operating area, monitoring its state using PIC microcontroller. 
The use of green energy is increasing, and electric vehicles are a better environmental option. Lithium-ion batteries are widely used due to their high energy density, but they require proper protection. Therefore, a Battery Management System (BMS) is essential to ensure safe operation.

In this project, a BMS using a PIC microcontroller is designed to monitor and protect the battery. The system includes a temperature sensor, current sensor, relays, charger, cooling fan, and LCD display. It continuously monitors battery voltage, current, and temperature. When the battery is low, it charges using relays in two modes: fast and slow charging. If the temperature exceeds a limit, the system activates a buzzer and cooling fan. The LCD displays all parameters in real time. The system ensures safe, efficient, and reliable battery operation.

The major building blocks of this project are:

· Regulated power supply 

· PIC Microcontroller.

· Temperature sensor.

· Voltage sensor.
· Current sensor.
· Buzzer. 
· LI-ION Battery pack

· TWO Relays.

· Charging Circuit.

· LCD display. 
· Cooling fan.
· LED Indicators 
· Crystal oscillator 
· Reset button.
Software’s used: 

· PIC-C compiler for Embedded C programming.

· PIC kit 2 programmer for dumping code into Micro controller.

·  Express SCH for Circuit design.

Block diagram:

[image: image3.jpg]



www.hvstechnologies.in                                                                            Ph: +91 9603140482

Ameerpet:      #A-7, 2nd floor, Eureka court, Above KS bakers, Ameerpet, Hyderabad – 73.


[image: image2.jpg]