Cloud integrated IoT framework for remote Parkinson's disease tracking

https://www.youtube.com/watch?v=kQsZGbIWVCg

Parkinson’s disease is a progressive neurological disorder characterized by motor symptoms such as tremors, rigidity, and bradykinesia, which require continuous monitoring for effective disease management. This project presents a Cloud Integrated IoT Framework for Remote Parkinson’s Disease Tracking, implemented in the form of a battery-operated smart watch for real-time and continuous health monitoring.

The proposed system employs a 9-axis gyroscope sensor (accelerometer, gyroscope, and magnetometer) to accurately capture hand motion data and quantify Parkinson’s tremors in terms of tremors per second. A TinyML-based machine learning model is deployed on the embedded device to analyze motion patterns locally, enabling efficient, low-power, and real-time tremor detection without constant cloud dependency. In addition to motion analysis, the system integrates a MAX30102 sensor to monitor vital physiological parameters such as heart rate and blood oxygen saturation (SpO₂), while body temperature is also measured for comprehensive health assessment.

All real-time sensor readings, including tremor frequency, heart rate, SpO₂, and temperature, are displayed locally on an OLED display for immediate user feedback. Simultaneously, the collected data is transmitted to the ThingSpeak cloud platform, allowing remote access, visualization, and long-term data analysis by healthcare professionals and caregivers. The cloud integration enables trend monitoring, early detection of symptom progression, and data-driven clinical decision support.

The compact, wearable, and low-power design of the smart watch ensures user comfort and continuous operation, making it suitable for daily use by Parkinson’s patients. Overall, the proposed system offers a cost-effective, scalable, and intelligent solution for remote Parkinson’s disease monitoring, enhancing patient care through real-time analytics, cloud connectivity, and embedded intelligence using TinyML.

Objectives:

- To design and develop a battery-operated smart watch for continuous and non-invasive monitoring of Parkinson’s disease symptoms.

- To accurately detect and quantify hand tremors (tremors per second) using a 9-axis gyroscope sensor for effective assessment of motor abnormalities.

- To implement a TinyML-based on-device intelligence model for real-time tremor analysis with low latency and reduced power consumption.

- To monitor key physiological parameters including heart rate and SpO₂ using the MAX30102 sensor and body temperature for comprehensive patient health tracking.

- To display real-time sensor readings and system status on an OLED display for immediate user awareness.

- To enable cloud integration using the ThingSpeak platform for remote data storage, visualization, and long-term trend analysis.

- To support remote monitoring by doctors and caregivers, facilitating early detection of disease progression and timely medical intervention.

- To ensure a compact, wearable, and user-friendly design suitable for daily use by Parkinson’s patients.

- To provide a cost-effective and scalable IoT-based healthcare solution for remote neurological disorder monitoring.

- To enhance patient quality of life through continuous monitoring, data-driven insights, and improved clinical decision support.

The major building blocks of this project are:

- Battery Power supply.

- ESP32.

- 9 Axis Gyroscope.

- MAX30102.

- Temperature sensor.

- OLED.

Software’s used:

- ARDUINO IDE.

- Embedded C language.

- Machine learning.

- Express SCH for Circuit design.

Block Diagram: